# RC3 - Command Station Commissioning Plan

# Overview

The RC3 is an integrated DCC Command station with booster, RailCom cutout and RailCom channel 2 global detector. It uses analogue comparators (LV393) and logic gates (74HC02) to detect and process RailCom signals. These are interfaced with a Raspberry Pi Pico or Pico2 with or without wireless.

# Pre-Commissioning Checks

| ## Visual Inspection |  |  |  |
| --- | --- | --- | --- |
| Note: Not all these are applicable to self-soldered boards. E.g. orientation etc should have been checked prior to soldering. Board assembly should be complete at this stage but no external connections are required and the Pico should not be mounted. |  |  |  |
|  |  | Action | Expected Result |
|  |  | This script is for the RC3 Revision 1B or 1C. Confirm PCB name and revision on back of PCB. |  |
|  |  | Inspect PCB for manufacturing defects (cracks, lifted pads, solder bridges) |  |
|  |  | Check for correct component orientation (especially ICs and polarised components) |  |
|  |  | Verify all components are soldered flush against the PCB |  |
|  |  | Inspect for cold or dry solder joints, particularly around fine-pitch components (e.g. U1 & U2), DCC bypass diodes (D1 & D2) Note: Bypass diodes carry the bulk of the DCC traction current. Poor or open circuit joints will impair DCC power delivery and also expose IC inputs to potentially damaging voltages. |  |
|  |  | Check that no solder bridges exist between signal traces |  |
| ## Unpowered Checks - Pico not inserted/mounted |  |  |  |
|  |  | Action | Expected Result |
|  |  | Measure DC resistance between +12 V and GND. Repeat for 5 V & GND and 3V3 & GND. The latter two are easily accessible on the SPI1 header (J9). | +12V and 3V3 should be >1 MΩ with no applied power. 5V will be c. 5 kΩ |
| ## Power-Up Checks - Pico not inserted/mounted |  |  |  |
|  |  | Apply 12V power to ‘DC IN’ (J5) observing polarity. If you have a bench power supply you can ramp the voltage up slowly to 12V and limit the current to 100mA. Monitor for: - Excessive current draw (if measurable) - Smoke or burning smell - Component heating (especially U5) |  |
|  |  | Measure voltage at 5V (SPI1 pin 1) | Should be 5V ± 0.2V |
|  |  | Measure voltage at +3V3 (SPI1 pin 2) | Should be 0V. |
|  |  | Disconnect 12 V supply. |  |
| ## Power-Up Checks - Pico inserted/mounted |  |  |  |
|  |  | Note: Use battery powered instruments. When DC booster power is connected, voltage differences on the board may approach booster 'DC in' 2. Take care. |  |
|  |  | Apply power to the Pico USB connector. Avoid risk to PC by using wall wart. Alternatively if using a PC put a USB isolator in circuit. |  |
|  |  | Measure voltage at 5V (SPI1 pin 1) | Should be at USB bus voltage. |
|  |  | Measure voltage at +3V3 (SPI1 pin 2) | Should match spec (typically 3.3V ±10%). |
|  |  | Disconnect USB power and connect 12 V DC supply. |  |
|  |  | Measure voltage at 5V (SPI1 pin 1) | Should be 5V ± 0.2V |
|  |  | Measure voltage at +3V3 (SPI1 pin 2 | Should match spec (typically 3.3V ±10%) |

# Component Testing

Prerequisites:

- Pico, Pico W, Pico2 or Pico2 W pre-loaded with MicroPython 1.26 or later inserted/mounted.

- USB connection to PC running Thonny (or other MicroPython capable IDE). Script instructions assume Thonny. Do not connect yet.

- MicroPython test application "tst_lcl_bd.py" available on PC

- DCC command station, booster and RailCom cutout available

No MicroPython application need be loaded at this stage. If there is a ‘main.py’ at the top level of the Pico’s directory structure it’s preferable to delete it. The MicroPython runtime will try to run it in preference to talking to you!

Note: USB power to Pico is sufficient for these tests. Do not connect board's 12V DC supply as this may create a ground loop, possibly resulting in damage to the board or PC.

| ## Static Tests |  |  |  |
| --- | --- | --- | --- |
|  |  | Action | Expected Result |
|  |  | Ensure booster power not connected (J4/J5). No track or other load should be connected to DCC out (J3). |  |
|  |  | Connect Pico to PC with USB and run Thonny. | Thonny 'Shell' window should display Pico start up message similar to: |

MicroPython v1.28.0 on 2026-04-06; Raspberry Pi Pico 2 W with RP2350

Type "help()" for more information.

|  |  | Use the Thonny 'Files' window to navigate to the PC directory containing ‘test_cs_bd.py’. Double click on it to open in the Thonny editor. |  |
| --- | --- | --- | --- |
|  |  | Run the program using 'Run Current Script' (Green play button or F5) | 'Shell' window should show: |

|  |  | Typing <return> will execute the next test. Typing n<return> will execute test n Test results are displayed on the Thonny shell screen and PCB LEDS as appropriate. Most tests run, display results and then terminate, awaiting next test initiation unless described otherwise. |  |
| --- | --- | --- | --- |
|  |  | Test 0 – Pico onboard LED. This test runs until the next test is started. | Pico onboard LED flashes at 1 Hz. |
|  |  | Test 1- D5 & D6 This test runs until the next test is started. | NeoPixels flash white at 1 Hz. |
|  |  | Test 2 – displays process unique id string and type as held by in build details. Check that build details match designation printed on Pico silkscreen. D5 shows red for unsupported processor, green for Pico, blue for Pico2. D6 shows blue if ‘W’ option. D5; D6; Proc. Type. Red; Unsupported. Green; Unlit; Pico. Blue; Unlit; Pico2. Green; Blue; Pico W. Blue; Blue; Pico2 W. | D5 green or blue – see table. D6 blue or unlit – see table. |
|  |  | Test 3 - global detector input GPIO pin state. Expected result is 0 (pin low) | D6 green. Red indicates error. |
|  |  | Test 4 – I2C tests. Checks I2C SDA & SCL pullups present. D5 shows red if no SDA pullup. D6 shows red if no SCL pullup. If pullups OK, checks OLED address (60) present. Shows green on both LEDs if present, else orange. | Both D5 & D6 show green. |
|  |  | Test 5 – DRV8874 asleep. Check fault and current sense. Pico ADC has a small offset ADC error at 0 current resulting in a non-zero reading. Brightness on D5 varies with current sense. D5 shows red if fault true and green if fault false. A reported current of c. 10 mA is OK. | D5 dim blue, D6 green. |
|  |  | Test 6 – DRV8874 awake. As above but fault is expected as booster power is off. | D5 dim blue, D6 red. |
|  |  | Note: the next tests require booster DC power to be connected at J4 or J5. The only guaranteed way to do this safely is to execute this test from a laptop running on battery – no mains connection. You may be OK with a USB isolation device and/or a low powered booster supply that has a floating DC power output but this is at your risk. The track should not be connected at this stage. |  |
|  |  | Repeat Test 5 with booster power connected. | D5 dim blue, D6 green. |
|  |  | Repeat Test 6 with booster power connected. In addition to the Pico zero offset error, the DRV8874 reports a small current when no load is attached. | D5 slightly brighter blue, D6 green. |
|  |  | Test 7 – Check detector power. This test runs continuously until terminated by typing Control + C. With booster power connected measure the voltage at D3 anode (terminal 2 – closer to board edge) with respect to GNDPWR (J4 terminal 2). N.B this will be -ve wrt to GNDPWR. | Should be –(booster DC supply volts – 2) |
| ## Reset Button |  |  |  |
|  |  | Press the reset button (SW1). | This should result in the following display in red on Thonny's 'Shell' window: |

Connection lost -- read failed: [Errno 6] Device not configured

Use Stop/Restart to reconnect.

Process ended with exit code 1.

# Operational Testing

This requires the Pico to be loaded with the MicroPython application software.  This may be obtained from the GitHub repository https://github.com/redhead-p/rp2da2. To install the software, follow instructions in the README.md file. Do not install main.py at this stage.

Prerequisites:

- As per Component Testing above

- Test track

- At least one loco fitted with a RailCom capable decoder

In addition to the power connection to ‘DC’ (J4 or J5) on the PCB connections are required to the track (J3).

The MicroPython scripts that supports operational testing is “test_dcccmd.py”. This is not to be installed on the Pico and may be run from the Thonny editor window using ‘Run current script’ as above. However these scripts make use of other modules in the suite and therefore the application software must be installed.

Note that both USB and power connections are required concurrently. Be aware of the note above concerning precautions to avoid equipment damage and ground loops.

| ## OLED Operation |  |  |  |
| --- | --- | --- | --- |
|  |  | Load the OLED driver “/lib/oled0_91.py” into the Thonny editor window. It has a simple self-test function. Run it with the Thonny ‘Run current script’. The Thonny shell shows the time in µs to display the screen. | The OLED displays a test splash screen. |

| ## Channel 2 detection |  |  |  |
| --- | --- | --- | --- |
|  |  | Load the script “test_dcccmd.py” into the Thonny editor window. With DC booster power off and no rolling stock on the test track, run it. | The OLED displays: DCC Test Both LEDs are unlit |
|  |  | Type: dcc.power(1) at the REPL prompt to power the track but note no DC power is connected. | D6 shows purple – no power fault condition. |
|  |  | Connect DC power. | D6 shows green. |
|  |  | Type: print_stats() | The Thonny shell should show something like: |

** Commands **

Rate: 96.27 per sec

Command packets: {'I': 17557}

** Channel 2 **

datagrams: set()

err. rate: 0%

|  |  | Place a loco with a RailCom compatible decoder on the track. Type: dcc.set_speed(n, 1, 0) where n is the dcc address of the loco. 1 is for forward motion and 0 is the speed. | The program responds: True showing the command is accepted. The loco doesn’t move. |
| --- | --- | --- | --- |
|  |  | Type: print_stats() The datagram ids received depend on the decoder. They vary according manufacturer, decoder model and release. | The Thonny shell shows that datagrams have been received. Details may differ from the example below. |

** Commands **

Rate: 62.57 per sec

Command packets: {'S': 1010}

** Channel 2 **

datagrams: {64, 7}

err. rate: 0%

|  |  | In the example above ‘64’ indicates an ACK. ‘7’ indicates a dynamic information datagram. For further details on the dynamic information datagram type: print_dyn_info() | Details of the received dynamic information are displayed. |
| --- | --- | --- | --- |
|  |  | Reverse the loco on the track and repeat previous three tests to confirm that RailCom channel 2 datagrams continue to be received. |  |

# Troubleshooting Guide

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| No power supply voltage | Regulator failure, shorted supply rail | Check regulator continuity, measure between VCC and GND, look for solder bridges |
| High idle current (>100mA) | Shorted component, incorrect value resistor | Measure individual power pins, check component orientation |
| Op-amp / comparator output always HIGH/LOW | Bad IC, supply not present, input shorted | Measure IC supply voltages, test with oscilloscope, check input circuit |
| No signal detection on one channel | Broken trace, bad amplifier, improper gain | Continuity check from input to output, replace op-amp / comparator, verify resistor values |
| Unreliable RailCom detection | Reference levels out of spec. | Check reference levels |

# Final Verification Checklist

After all testing is complete:

- Power consumption is within expected limits

- Temperature rise is acceptable (<10°C above ambient at full operation)

- Extended operational test (>1 hour) shows no drift or instability

- Board ready for integration into main system

# Test Equipment Required

- Oscilloscope (bandwidth >2 MHz recommended) – useful for fault diagnosis but not essential.

- Multimeter with impedance/resistance measurement

- Bench power supply (0-12 V, ≥500 mA) – useful but not essential or

- DC booster power supply.

- Logic analyzer (recommended for signal timing verification) – useful for fault diagnosis but not essential

- Test Track

- Locos fitted with RailCom decoders and with the relevant CVs set to enable RailCom. (Typically, this is the default setting.)

# Notes

- Document all measurements for future reference

- Keep record of any component replacements or rework