# RC_LCL 4 - Quad RailCom Detector Commissioning Plan

# Overview

The RC_LCL_4 is a four-channel RailCom detector board designed for DCC (Digital Command Control) track feedback. It uses analogue comparators (LV393LV) and logic gates (74HC02) to detect and process RailCom signals and amplifiers (TLV9062) for occupancy detection. These are interfaced with a Raspberry Pi Pico2 or Pico 2 W.

The board is split into two parts which are galvanically isolated. The split is obvious on the reverse side of the PCB as each part has its own ground plane. One part is 'track-side', the other is Pico side. Texas Instrument ‘ISO…’ series galvanic isolators transmit data signals across the galvanic divide.

Note: The PCB has three header strips J6, J8 and J4. These are available for external connections by application extension, but are not used as part of the default application and are not tested comprehensively here.

# Pre-Commissioning Checks

| ## Visual Inspection |  |  |  |
| --- | --- | --- | --- |
| Note: Not all these are applicable to self-soldered boards. E.g. orientation etc should have been checked prior to soldering. |  |  |  |
|  |  | Action |  |
|  |  | This script is for the RC_LCL_4 Revision 0.C or 0.D PCB. Confirm PCB name and revision on back of PCB. |  |
|  |  | Inspect PCB for manufacturing defects (cracks, lifted pads, solder bridges) |  |
|  |  | Check for correct component orientation (especially ICs and polarised components) |  |
|  |  | Verify all components are soldered flush against the PCB - no tombstones! |  |
|  |  | Inspect for cold or dry solder joints, particularly around - fine-pitch components (e.g. U14, U34, RN1, RN2, RN11 & RN31), - DCC bypass diodes (D11 - D14 & D31 - D34) and - the DCC Sense divider resistor (R52). Note: Bypass diodes carry the bulk of the DCC traction current. Poor or open circuit joints will impair DCC power delivery and also expose IC inputs to potentially damaging voltages. Poor or open circuit joints for R52 will expose IC inputs to potentially damaging voltages |  |
|  |  | Check that no solder bridges exist between signal traces |  |
| ## Unpowered Checks - Pico not inserted/mounted |  |  |  |
| Note: Power and Track should not be connected. |  |  |  |
|  |  | Action | Expected Result |
|  |  | Measure DC resistance between +12 V and GND. Repeat for 5 V & GND and 3V3 & GND. All these are easily accessible on the SPI1 header (J8). | Should be >1MΩ with no applied power. |
|  |  | Measure DC resistance between Pico Ground (GND) and track-side ground (GND1) test points. | Should be open circuit with no measurable resistance. |
|  |  | Measure DC resistance at DCC connector (J3). Measured value will vary as C1 or C2 charges. | Should not be closed circuit. It should approach something > 100K |
| ## Power-Up Checks - Pico not inserted/mounted |  |  |  |
|  |  | Apply 12V power to ‘DC IN’ (J5) observing polarity. If you have a bench power supply you can ramp the voltage up slowly to 12V and limit the current to 100mA. Monitor for: - Excessive current draw (if measurable) - Smoke or burning smell - Component heating (especially U5) | Led D9 should light. |
|  |  | Measure voltage at 5V (SPI1 pin 2) | Should be 5V ± 0.2V |
|  |  | Measure voltage at +3V3 (SPI1 pin 10) | Should be 0V. |
|  |  | Disconnect 12 V supply. |  |
| ## Power-Up Checks - Pico inserted/mounted |  |  |  |
|  |  | Apply power to the Pico USB connector. Avoid risk to PC by using wall wart. If using a PC I'd use a USB isolator in circuit, but these might just be snake oil. | Led D9 should light. |
|  |  | Measure voltage at 5V (SPI1 pin 2) | Should be at USB bus voltage. |
|  |  | Measure voltage at +3V3 (SPI1 pin 10) | Should match spec (typically 3.3V ±10%). |
|  |  | Disconnect USB power and connect 12 V DC supply. |  |
|  |  | Measure voltage at 5V (SPI1 pin 2) | Should be 5V ± 0.2V |
|  |  | Measure voltage at +3V3 (SPI1 pin 10) | Should match spec (typically 3.3V ±10%) |
|  |  | Disconnect 12 V supply. |  |
| ## Power-Up Checks - DCC/'track-side' |  |  |  |
|  |  | Note: Polarity of DCC connection is not significant at this stage so it's not necessary to resolve which is Left and Right if booster output not labelled in these terms. Note: When DCC power is connected, booster ground is not same as GND1. Use battery powered instruments. Voltage differences on 'track-side' part of board may approach booster 'DC in' 2. Take care. Pico side power (USB or 12V DC) should not be connected for these tests. |  |
|  |  | With booster power off, connect booster DCC out to ‘DCC IN’ (J3). Preferably power booster from 12V DC. |  |
|  |  | Turn DCC power on at booster. Monitor for: - Excessive current draw (if measurable) / booster over current indication - Smoke or burning smell - Component heating |  |
|  |  | Measure voltage at C1 +ve terminal wrt C2 -ve terminal. | Should approach 2 booster 'DC in' after allowing for booster losses and on-board diode voltage drops. |
|  |  | Measure voltage at DCC-L test point wrt GND1. | Should be +2.5V. |
|  |  | Measure voltage at +5VA wrt GND1. +5VA is at R1 terminal 2 - the end closer to R5/centre of board. | Should be 5 V. |
|  |  | Measure voltage at Upper_Ref wrt DCC-L. Upper_Ref is at R6 terminal 1 - the end closer to U1. | Should be 14 mV ±5% |
|  |  | Measure voltage at Lower_Ref wrt DCC-L. Lower_Ref is at R5 terminal 1 - the end closer to R1 | Should be -14 mV ±5%. |
|  |  | Measure the voltage either side of R54 wrt DCC-L. | Voltages should be 250 mV and -250 mV, each ±10%. |

# Component Testing

Prerequisites:

- Pico2 or Pico2 W pre-loaded with MicroPython 1.26 or later inserted/mounted.

- USB connection to PC running Thonny (or other MicroPython capable IDE). Script instructions assume Thonny. Do not connect yet.

- MicroPython test application "tst_lcl_bd.py" available on PC

- DCC command station, booster and RailCom cutout available

No MicroPython application need be loaded at this stage. If there is a ‘main.py’ at the top level of the Pico’s directory structure it’s preferable to delete it. The MicroPython runtime will try to run it in preference to talking to you!

Track should not be connected at this stage.

Note: When DCC power is connected, Pico ground (GND) is not same as GND1.

Note: USB power to Pico is sufficient for these tests. Do not connect board's 12V DC supply as this may create a ground loop, possibly resulting in damage to the board or PC.

| ## Static Tests |  |  |  |
| --- | --- | --- | --- |
|  |  | Action | Expected Result |
|  |  | Ensure booster not connected. |  |
|  |  | Connect Pico to PC with USB and run Thonny. | Thonny 'Shell' window should display Pico start up message similar to: |

MicroPython v1.28.0 on 2026-04-06; Raspberry Pi Pico 2 W with RP2350

Type "help()" for more information.

|  |  | Use the Thonny 'Files' window to navigate to the PC directory containing ‘tst_lcl_bd.py’. Double click on it to open in the Thonny editor. |  |
| --- | --- | --- | --- |
|  |  | Run the program using 'Run Current Script' (Green play button or F5) | 'Shell' window should show: |

RailCom Quad Local Detector Commissioning Tests

0 Flash Onboard LED

1 Flash NeoPix

2 Check processor

3 Check User Switch

4 Check I2C 0

5 Check I2C 1

6 Check rx pins

7 Check or pins

8 DCC Sense

>

|  |  | Typing <return> will execute the next test. Typing n<return> will execute test n. The program will exit after completing the last test. Some tests return the invitation to type ‘>’immediately. Other need to be terminated by typing ‘Control + C’. Hint: if you are running Thonny on a Mac then ‘Control + C’ means ‘Control + C’ not ‘Command + C’! Test results are displayed on the Thonny shell screen and PCB LEDS as appropriate. |  |
| --- | --- | --- | --- |
|  |  | Test 0 – Pico onboard LED. | Pico onboard LED flashes at 1 Hz. |
|  |  | Test 1- D5 & D6 | NeoPixels flash white at 1 Hz. |
|  |  | Test 2 – displays process unique id string and type as held by in build details. Check that build details match designation printed on Pico silkscreen. D5 shows red for unsupported processor, green for Pico, blue for Pico2. D6 shows blue if ‘W’ option. Note: this board requires a Pico2 or Pico2 W D5; D6; Proc. Type. Red; Unsupported. Green; Unlit; Pico. Blue; Unlit; Pico2. Green; Blue; Pico W. Blue; Blue; Pico2 W. | D5 blue. D6 blue or unlit as appropriate. |
|  |  | Test 3 – tests the user switch (SW2). Control + C to stop test. | With switch released: D5 shows RED Pico built in LED unlit With switch pressed: D5 is unlit Pico built in LED is lit |
|  |  | Test 4 – I2C0 tests (OLED). Checks I2C0 SDA & SCL pullups present. D1 shows green if SDA pullup OK. D2 shows green if SCL pullup OK. OLED address (60) and ADC addresses (72 & 73) are checked. If present D3, D4 & D5 respectively show blue. If any of the above fail, the relevant LED shows red. Note: addresses 72 & 73 are on I2C1 and are not expected here as booster is not connected. | D1 & D2 show green. D3 shows blue. D4 & D5 show red. |
|  |  | Test 5 – I2C1 tests (ADC). Checks I2C1 SDA & SCL pullups present. D1 shows green if SDA pullup OK. D2 shows green if SCL pullup OK. OLED address (60) and ADC addresses (72 & 73) are checked. If present D3, D4 & D5 respectively show blue. Note: booster not connected so addresses 72 & 73 not active and OLED is on other I2C bus. | D1 & D2 show green. D3, D4 & D5 show red. |

|  |  | Test 6 - Test local detector input receive GPIO pin states. D1 to D4 show states of GPIO14, 16, 18, 20 respectively. Green for high, unlit for low. This test runs until terminated with Control + C. | D1 to D4 unlit and steady. Most recent states shown on test termination as below. |
| --- | --- | --- | --- |

Pin(GPIO14, mode=IN) 0
Pin(GPIO16, mode=IN) 0
Pin(GPIO18, mode=IN) 0
Pin(GPIO20, mode=IN) 0

|  |  | Test 7 - Test local detector input orientation GPIO pin states. D1 to D4 show states of GPIO15, 17, 19, 21 respectively. Green for high, unlit for low. This test runs until terminated with Control + C. | D1 to D4 green and steady. Most recent states shown on test termination as below. |
| --- | --- | --- | --- |

Pin(GPIO15, mode=IN) 1

Pin(GPIO17, mode=IN) 1

Pin(GPIO19, mode=IN) 1

Pin(GPIO21, mode=IN) 1

|  |  | Note: As the DCC side is unpowered any discrepancies indicate faults on the Pico side of the board. These should be investigated and remedied before proceeding further. I2C pins have pull-up resistors. A value of ‘0’ on an I2C pin suggests a problem here. |  |
| --- | --- | --- | --- |
|  |  | Connect DCC power (J3) and turn booster on. Repeat Test 5 above. ADC should now be detectable on I2C1. | D1 & D2 show green. D3 shows red D4 & D5 show blue. |
|  |  | Repeat Test 6. Receive pins should now be high. | D1 to D4 green and steady. Most recent states shown on test termination as below |

Pin(GPIO14, mode=IN) 1

Pin(GPIO16, mode=IN) 1

Pin(GPIO18, mode=IN) 1

Pin(GPIO20, mode=IN) 1

|  |  | Repeat Test 7. Orientation pins should now be low. | D1 to D4 unlit and steady. Most recent states shown on test termination as below |
| --- | --- | --- | --- |

Pin(GPIO15, mode=IN) 0

Pin(GPIO17, mode=IN) 0

Pin(GPIO19, mode=IN) 0

Pin(GPIO21, mode=IN) 0

|  |  | Note: A discrepancy here could indicate a problem on the Pico or DCC track-side. As before, this needs investigation and remedy before proceeding. |  |
| --- | --- | --- | --- |
|  |  | The following test each of the four block connections. Note: Resistor values in following tests are for booster running off 12V DC supply. Adjust if using a 15V supply |  |
|  |  | Input Impedance: This checks integrity of DCC Traction current path too. With DCC Power from booster disconnected, measure input resistance DCC_LEFT to DCC_BLK_n with multimeter (J3 terminal 2 to J1/2 terminal 1/2 as appropriate). This measurement is not affected by DCC booster supply voltage. | Should be 1.8 Ω after allowing for resistance of multimeter leads. |
| ## RailCom Detection Circuit (Analogue Front-End) |  |  |  |
|  |  | Connect DCC power from booster. |  |
|  |  | Over RailCom Threshold: Run test 6 to monitor the detector receive pins. Connect a 1 kΩ resistor from DCC_RIGHT (J3 Terminal 1) to DCC_BLK_n (n = 1 to 4). With the resistor disconnected all LEDs should be green. With the resistor connected relevant LED will be mostly unlit but flickering. This is because the scanning is not synchronised with the DCC signal. | Check that the LED corresponding GPIO Pin 14, 16, 18 or 20 as appropriate reacts to the resistor. Check that other LEDs do not change (still green). |
|  |  | Run test 7 to monitor the detector orientation pins. Repeat the test on each DCC_BLK_n terminal with the 1 kΩ. With the resistor disconnected all LEDs should be unlit. With the resistor connected the relevant LED will flicker, but more brightly than above. As above scanning is not synchronised with the DCC signal. | Check that the LED corresponding GPIO Pin 15, 17, 19 or 21 as appropriate reacts to the resistor. Check that other LEDs do not change (still unlit). |
|  |  | Under RailCom Threshold: Repeat above two tests with 2 kΩ resistor. | The LEDs should not change in reaction to the resistor being connected and should remain steady green or unlit. |
| ## DCC Power Sense |  |  |  |
|  |  | With DCC power connected run test 8 to check for cutout detection. The function will sample the DCC_Sense input for 10s and count the number of cutouts spotted. This depends greatly on the command station. | Should be about 100 per sec. and no lower than 30. |
| ## Block Occupancy Detection |  |  |  |
|  |  | Conventional block occupancy detection uses op. amps on the track-side to amplify the voltages across the current sense resistors to a level suitable for ADC conversion. Two ADC converters monitor two sense resistors each. The ADC converters have I2C connections to the Pico via a galvanic I2C isolator. The ADC sample time is comparatively long compared with DCC bit times and is not synchronised with DCC transmissions. Raw values may be +ve or -ve and samples may overlap DCC polarity changes or cutouts. There is an error due to op. amp. input bias. The main application takes multiple samples, applies corrections for input offset bias and filters the raw readings. |  |
|  |  | Blocks Unoccupied: Block connectors should have no connections.. Use test 9 – ADC – Single Shot to read ADC outputs. | You should see something like: |

1 20

2 23

3 16

4 -11

5 0

6 1249

7 0

8 1250

|  |  | Readings 1 to 4 are the differential readings across the sense resistors wrt DCC-L. They are non-zero due to input bias at the op. amps. The units are mV. Readings 1 to 4 are saved as zero reference values for the next test. | Should be 0 ±25 mV |
| --- | --- | --- | --- |
|  |  | Readings 5 & 7 are differential readings across ADC inputs which are both at DCC-L. | Should be 0 or very close. |
|  |  | Readings 6 & 8 are single ended readings of DCC-L wrt to ADC ground (GND1). The units are 2 mV. | Should be 1250 ±2% (i.e. 2.5 V ±2%) |
|  |  | Blocks Occupied 10K: 10 kΩ resistors are typically used for block occupancy detection wheel sets. With DCC power and USB connection run test 10 – ADC Continuous. No resistor should display orange. Check that a 10 kΩ resistor connected from DCC_RIGHT (J3 Terminal 1) to DCC_BLK_n (n = 1 to 4) results in a display on the relevant LED. There may be some flickering. This is acceptable. | A 10 kΩ resistor should give a typically reading in the range ±(50 to 150) which will result in a green colour. |
|  |  | Blocks Occupied 1K: Repeat with a 1 kΩ resistor. This is roughly equivalent to a quiescent mobile decoder. Check that a 1 kΩ resistor connected from DCC_RIGHT (J3 Terminal 1) to DCC_BLK_n (n = 1 to 4) results in an appropriate value for readings 1 to 4 using the 'test_adc()' function. | A 1 kΩ resistor should give a reading in the range ±(500 to 1500) which will result in a blue colour. Green may be displayed for a short time when the resistor is removed. |
| ## Reset Button |  |  |  |
|  |  | Press the reset button (SW1). | This should result in the following display in red on Thonny's 'Shell' window: |

Connection lost -- read failed: [Errno 6] Device not configured

Use Stop/Restart to reconnect.

Process ended with exit code 1.

# Operational Testing

This requires the Pico to be loaded with the MicroPython application software.  This may be obtained from the GitHub repository https://github.com/redhead-p/rp2da2. To install the software, follow instructions in the README.md file. Do not install main.py at this stage.

Prerequisites:

- As per Component Testing above

- Test track with four independent blocks

- At least one loco fitted with a RailCom capable decoder

In addition to the booster connection to ‘DCC In’ (J3) on the PCB connections are required to the track. Blocks must be insulated from each other by a break in the same rail and have independent feeder wires. The other rail is the common rail. Each detector circuit measures the current flowing in the DCC-Left connection as identified on the PCB. The common rail is connected to DCC-Right as identified on the PCB and each block’s feeder is connected to DCC Out Blk1, Blk2, Blk3 or Blk4 (J1 & J2).

There are two MicroPython scripts that support operational testing: “test_dccrc1.py” and “test_quad_blk.py”. These are not to be installed on the Pico and may be run from the Thonny editor window using ‘Run current script’ as above. However these scripts make use of other modules in the suite and therefore the application software must be installed. These scripts will run until terminated by Control + C or other Thonny command.

| ## Conventional Block Occupancy Detection |  |  |  |
| --- | --- | --- | --- |
|  |  | Load the script “test_quad_blk.py” into the Thonny editor window. With DCC booster power off and no rolling stock on the test track, run it. | All block status LEDs (D2 – D5) should show red indicating no DCC power to the detector. The REPL log will show event codes 23 for each block. |
|  |  | Turn the DCC power on. | The LEDs should all go out. The REPL log will show event codes 20 for each block. |
|  |  | Place a loco with a RailCom decoder on one of the blocks. | The relevant status LED should show green. All others remain unlit. The REPL log will show event code 22 for the relevant block. |
|  |  | Remove the loco. | All status LEDs should be unlit. |
|  |  | Repeat for each of the blocks in turn. | The relevant status LED should show green. All others remain unlit. The REPL log will show events as they occur. |
|  |  | Use the command station to move the loco under its own power to the extent permitted by the test track. | The lit status LED should follow the movements. Transits between blocks will have period of overlap when both are lit. The REPL log will show events as they occur |
|  |  | Repeat above steps as applicable with 10 kΩ wheel set equipped item of rolling stock (or failing that you can use a 10 kΩ resistor direct) | As above. |
|  |  | Type Control + C to terminate the script. Set the loco speed to 0 and turn DCC power off. |  |
| ## RailCom Block Occupancy Detection |  |  |  |
|  |  | Load the script “test_dccrc1.py” into the Thonny editor window. With DCC booster power off and no rolling stock on the test track, run it. As for the above tests the REPL log will show events as they occur. | All block status LEDs should be unlit. |
|  |  | Place a loco with a RailCom decoder on one of the blocks. Set the loco speed to 0. Turn the DCC power on. | The relevant status LED should show blue. All others remain unlit. The line of the OLED display coresponding to the block should show the block name, DCC decoder address type (short, long or consist), address value and its orientation (either 1 or -1). E.g. something like. |

Blk1 s10 1

|  |  | Press the user button. | Statistics are printed on the Thonny ‘Shell’ window. E.g. |
| --- | --- | --- | --- |

** Channel 1 Blk1 **

Msg. rate: 0.00 per sec

** Channel 1 Blk2 **

Msg. rate: 0.00 per sec

** Channel 1 Blk3 **

Msg. rate: 75.49 per sec

err. rate: 0%

** Channel 1 Blk4 **

Msg. rate: 0.00 per sec

Note that pressing the user button resets the statistics after they have been printed. If stationary the error rate should be very low e.g. 0% as above.

Crossing block boundaries will increase the error rate significantly as result of there being indeterminacy about which detector the current flows through, there being fluctuations in resistance caused by varying contact resistance as wheels etc rotate. The following shows errors that have occurred when crossing the block boundary between blocks 3 and 4.

** Channel 1 Blk1 **

Msg. rate: 0.00 per sec

** Channel 1 Blk2 **

Msg. rate: 0.00 per sec

** Channel 1 Blk3 **

Msg. rate: 15.70 per sec

W_HIGH	1

DG_INCOMP	1

err. rate: 4%

** Channel 1 Blk4 **

Msg. rate: 65.27 per sec

W_HIGH	4

CB_IN_DG	1

err. rate: 2%

|  |  | Remove the loco. | The relevant status LED should be unlit. All others remain unlit. Loco details will be removed from the OLED. |
| --- | --- | --- | --- |
|  |  | Turn the loco around and replace on the track. | The relevant status LED should show blue. The bottom line of the OLED display should identify the block etc. as before but with 1 swapped for -1 or vice versa. |
|  |  | Set the command station to move the loco under its own power to the extent permitted by the test track. | The lit status LED should follow the movements. Transits between blocks will have period of overlap when both are lit. The OLED will display block status change messages scrolling up from the bottom. |
|  |  | Type Control + C to terminate the script. Set the loco speed to 0 and turn DCC power off. |  |

# Troubleshooting Guide

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| No power supply voltage | Regulator failure, shorted supply rail | Check regulator continuity, measure between VCC and GND, look for solder bridges |
| High idle current (>100mA) | Shorted component, incorrect value resistor | Measure individual power pins, check component orientation |
| Op-amp / comparator output always HIGH/LOW | Bad IC, supply not present, input shorted | Measure IC supply voltages, test with oscilloscope, check input circuit |
| No signal detection on one channel | Broken trace, bad amplifier, improper gain | Continuity check from input to output, replace op-amp / comparator, verify resistor values |
| Cross-channel interference | PCB layout issue, improper grounding | Review ground plane continuity, check return path routing |
| Unreliable RailCom detection | Reference levels out of spec. | Check reference levels and DCC side voltages. |

# Final Verification Checklist

After all testing is complete:

- All 4 channels detect RailCom signals independently

- No cross-channel interference detected

- Power consumption is within expected limits

- Temperature rise is acceptable (<10°C above ambient at full operation)

- Extended operational test (>1 hour) shows no drift or instability

- Board ready for integration into main system

# Test Equipment Required

- Oscilloscope (bandwidth >2 MHz recommended) – useful for fault diagnosis but not essential.

- Multimeter with impedance/resistance measurement

- Bench power supply (0-12 V, ≥500 mA) – useful but not essential

- DCC track power supply e.g. Command Station / Booster / Cutout. Must be RailCom compatible. It must have a manual or virtual throttle to permit switching track power off and on and set speed/direction.

- Logic analyzer (recommended for signal timing verification) – useful for fault diagnosis but not essential

- Test Track – with four blocks

- Locos fitted with RailCom decoders and with the relevant CVs set to enable RailCom. (Typically, this is the default setting.)

# Notes

- Document all measurements for future reference

- Keep record of any component replacements or rework